How-To-Use

Environment Configuration

- Go to my SPM repo: https://github.com/AdiMatam/Battery-Pybamm/tree/sei-pack

  - Download repository as a ZIP file and extract to any directory on your machine

- Install VSCode: https://code.visualstudio.com/

  - Default installation settings

- Launch the editor and identify the column of icons on the left side (see pic below)

- Install Python extension

- Open the downloaded repository in VSCode

  - A file explorer dialog will open when you select “Open Folder…”. Navigate to the FOLDER from Step 1 and click “Select Folder”

- You should see something like this:

You MAY have to click on the orange circle to see the list-out of files in this directory.

- Press CTRL + ~. This is the button to the left of the 1 key. A terminal window will appear, run the following:

  - pip install -r requirements.txt

  - python mainmodel.py

MAIN FILES (That concern running the sim)

mainmodel.py: ’Main’ program which initializes Packs or Cells – runs simulation, cycling and any plotting/post-processing routines

- See the VARIABLES IN ALL CAPS AT THE TOP this program

params.py: Defines cell-level parameters (see ‘VariedParameter’)

- POS_ELEC_POROSITY   = VariatedParameter.from_percent(0.50, 5)

  - The positive electrode’s porosity will be 0.5 +/- 5% across all cells in pack

Top Down Look:

pack.py: Builds X Cells in series, Y Cells in parallel (X, Y are input params).

- Builds the DAE for the pack – all algebraic equations for coupling

- Includes discretization method and cycling protocol

cell.py: Couples 2 Single_Particles (which consistute one ‘Cell’)

- Cathode called Positive Electrode (+)

- Anode called Negative Electrode (-)

single_particle.py: Defines model of a single electrode.

- Diffusion ODEs, boundary/initial conditions for Lithium concentration, and Butler-Volmer calculations for electrode potential

- Global meshing geometry, parameter structures are updated

- Note that this program can be run standalone to validate the ‘charge’ / ‘discharge’ of a single particle.